Voyager Air Firmware Release 2.3

Version 2.3.0

This firmware release was developed to accompany Release 2.3 of the app software, both for iOS and Android. It is compatible with both the Voyager Air and Voyager Air 2 drives.

What is in this Release:

- Bug fixes related to Google Drive support

- Compatibility fixes for the iOS 8 version of the app

Before You Install:

- This release requires that firmware release 2.0 (version 2.0.17) or higher already be installed. If you have a Voyager Air 2 drive, v2.0.17 is the version it ships with.

- It’s recommended that you connect DC power to the drive during this update. If you don’t have DC power, please make sure your battery has at least 20% charge (and more is better). Running out of battery power during a firmware update risks turning the drive into a “brick.”

To Install From the App:

- Enable Wireless Passthru or Ethernet Passthru to connect your drive to the internet.

- Running the Mobile App or Web Client, go to “Settings > Firmware update” and press “Check”

- Follow the instructions in the app to download and install the update.

To Install Manually:

- Connect the drive  to your computer with the USB cable, and turn the drive on.

- Copy the file “VoyagerAir-2.3.0.pkg” into the top-level folder (root) on your drive.

- Eject and turn off the drive, then disconnect the USB cable (the update will not install if USB is still connected).

- Turn the drive on again.

- Connect to the drive, and launch the app or web client.

- In Settings > Drive Firmware you should see “Update Available” – press “Update” to start the firmware update process.

- You should soon see the Wi-Fi icon blinking, indicating that the update is being installed.

- When the icon stops blinking the update will be complete.

Remember to also download and install the Release 2.3 version of the App (from iTunes, Google Play or the Amazon App Store) onto your mobile device.

Note: Running this firmware release with the earlier versions (2.0, 2.1 or 2.2) of the iOS or Android App should present no major issues. You will see a message suggesting that you update the app, and some of the new features detailed above will not be available, but all major features will continue to work.